Get NVMe support on unsupported bios with BIOS modding

Topic

- BIOS MODIFY

My testing system

- Motherboard AsRock H77 Pro4/MVP

- CPU Intel i5-3570

My motherboard is not support NVMe in PCI slot by factory so I had to modified BIOS by my self

1.BIOS MODIFY

What you need

- Your BIOS file (get from your mainboard’s website)

- UEFITool.exe

- NvmExpressDxe_4.ffs

Step

- Open UEFITool.exe

- Click File > Open image file…

- Select your BIOS file

- Click on Intel > BIOS region

- Click File > Search

- Click on Text tab > Search for “DXE”

- Double click on the last one

- Scroll to the first on sub-folder

- Scroll to the last one before free space

- Click Insert after…

- Select NvmExpressDxe_4.ffs

- It should look like this

- Click at File > Save image file…

- I had to remove .mod file extensions as my unmodified BIOS does’t have

- DONE FOR MODIFY BIOS STEP

2.BIOS FLASHING

- Boot into BIOS setting > Instant Flash

- Select your BIOS file

- Wait for it

- Reboot